Assignment 3

| HC-SR04 Ultrasonic Distance Sensor | SG90 Micro Servo Motor |
| --- | --- |
| - What it does: It measures the distance between the sensor and an object by emitting an ultrasonic sound wave and timing how long it takes for the echo to bounce back. - Electronics/hardware inside: It contains an ultrasonic transmitter (a piezoelectric speaker that emits the sound), an ultrasonic receiver (a piezoelectric microphone that "hears" the echo), and a small control IC (integrated circuit) to process the timing of the signals. - Analog or digital: Digital. It uses a digital pulse to trigger the sound wave, and outputs a digital pulse where the length of the pulse represents the distance. - Units and range: Measures distance in centimeters (cm) or inches. The typical effective range is from 2 cm to 400 cm. | - What it does: It provides precise rotational movement. Unlike a standard motor that spins continuously, a servo can be commanded to turn to a specific, fixed angle and hold that position (commonly used for steering, robotic arms, or opening small valves). - Electronics/hardware inside: It consists of a small DC motor, a reduction gear train (to increase torque), a potentiometer (a variable resistor connected to the output shaft to monitor its current angle), and a small control board to compare the current angle with the requested angle. - Analog or digital: Digital. It is controlled using a digital PWM (Pulse Width Modulation) signal from the microcontroller. - Units and range: Movement is measured in degrees (°) of rotation. The typical range of motion for this servo is 0° to 180°. |